Emergency Evacuation Procedures

Objectives and scope:

It is the objective of Dorking Halls (MVDC) to ensure that all emergency situations are dealt with in a way that minimises risk to customers, staff and contractors.

These procedures identify the action to be taken in event of various emergency situations.  Staff must be fully trained in these procedures and copies are available as per the distribution list below.

Index

- Fire protection and alarm system:

- Emergency evacuation procedures:

- Emergency Evacuation for Disabled users at Dorking Halls

- Controlled evacuation procedures:

- Miscellaneous

1.0 Fire Alarm System

Dorking Halls has a 23-zone pre-alert analogue addressable (L2) fire alarm system with Break Glass call points and Automatic Fire Detection.  The system incorporates audible and visual warning and is supplemented by the use of the tannoy system.  The system is linked via Red Care to an ADT call centre.

  - Emergency Lighting

Dorking Halls is protected by a combination of maintained and non maintained emergency escape lighting.  This will ensure that in the event of a power cut there is sufficient light to enable occupants to exit the building safely. The emergency lighting in Dorking Halls will give a minimum of 3 hours operation.

  - Fire Fighting Equipment

Dorking Halls has a variety of strategically placed fire extinguishers and fire blankets to enable suitably trained members of staff to tackle small fires. See appendix (iiii) for the location of fire extinguishers / fire blankets in Dorking Halls.

1.3 Responsibility of the Designated Fire Investigator

Dorking Halls can be presented with a number of different scenarios for when an evacuation can take place. The role of designated Fire Investigator shall be distributed at the start of each shift and it is imperative for the Duty Manager to explain what the routines will be at the start of every shift change.

  - Action on discovering a fire

IF YOU DISCOVER A FIRE

- Immediately operate the nearest Alarm

- Contact the Duty Manager

- Leave the building by the nearest fire exit and report to the Assembly point

- Only attempt to fight a small fire with the correct extinguisher if you have been trained and can do so without putting yourself at risk. If instructed to evacuate by a fire marshal you must do so.

STAFF SHOULD NOT PUT THEMSELVES AT RISK BY FIGHTING A FIRE, IT IS MORE IMPORTANT TO RAISE THE ALARM.

WHAT TO DO WHEN THE ALARM SOUNDS

The Fire Alarm is a loud continuous electronic system.

If the alarm sounds be aware and ready to evacuate the building if necessary. Please wait for further instructions from the Duty Manager who will inform you either in person or on the speaker system on the requirement to evacuate or if it has been a False Alarm.

2.0 Emergency Evacuation

Fire is not the only situation in which it may become necessary to evacuate the building, other possible scenarios are: structural failure, bomb threat, serious gas escape.  This list is not exhaustive but serves to indicate that the emergency evacuation procedures can be used whenever it is necessary to get people out of the building without delay.

If an evacuation is necessary then leave the building by the nearest safe exit or as directed by one of our Fire Marshalls and assemble in the designated Assembly point. Do not stop to collect personal belongings.

Do not delay evacuation. DO not re-enter the building until told to do so.

2.1 Assembly Point

THE ASSEMBLY POINT FOR DORKING HALLS CUSTOMERS AND STAFF IS IN THE REIGATE ROAD CAR PARK TO THE LEFT OF THE St. JOHNS AMBULANCE BUILDING, as denoted by the ASSEMBLY POINT SIGN on the lamppost.

Customers / staff who are evacuated from the front of the building will need to go around the side of the sports centre and into the car park via the car park access road. It is likely that the sports centre will be evacuating at the same time (our emergency (fire) alarms are linked) so do not direct customers via the sports centre “street”, this will be one of the main routes for evacuating the sports centre and will be busy.

3.0 Emergency Evacuation for Disabled users at Dorking Halls

The evacuation of persons with disabilities is the responsibility of such individuals and Dorking Halls has established this policy so that individuals with disabilities are informed regarding evacuation guidelines. Individuals with disabilities are advised to take every precautionary measure to help ensure their own safety in an emergency situation.

Grand Hall -

Accessibility and seating for all shows, events and cinema performances is from the Ground Floor and in every event the Wheelchair users are situated at the nearest point to an emergency exit to be allowed to exit first.

Masonic Hall -

Accessibility and seating for all shows, events and cinema performances is from the Ground Floor and in every event the Wheelchair users are situated at the nearest point to an emergency exit to be allowed to exit first.

Martineau Hall –

Accessibility and seating for all shows, events and cinema performances is either from the Ground Floor or the First Floor.

Ground Floor - Wheelchair users are situated at the nearest point to an emergency exit to be allowed to exit first.

First Floor – The Duty Manager must be made aware of any disabled users accessing the Martineau Hall via the First Floor. The user must then be given clear instructions as to the safe methods of escape available to them and the location of the refuge point. It cannot be assumed that a wheelchair user is unable to manage stairs or is comfortable using the evacuation chair, so the options available to them will vary. It should also be noted that due to the layout of the building, evacuation via different routes may be necessary depending on the scenario.

Due to the nature of the building there are some parts that are inaccessible to Wheelchair users due to the lack of escape facilities in the event of an emergency.

These areas are: -

- 2nd Floor Balcony

- 2nd Floor Conference Room

- Administration Offices

- Grand Hall Projector & Spot Light area

3.1 Shows/ Events/ Conferences & Cinema

- Whenever the fire alarms sound, all customers of the building should evacuate the building and assemble at the appropriate assembly points. Customers with disabilities are encouraged to evacuate the building and should remain with their group at the appropriate assembly point.

3.2 Events in the Mezzanine.

- The Duty Manager will be aware of any disabled users attending the function.

- LIFTS SHOULD NOT BE USED for evacuations.

- If an evacuation is required the wheel chair user should make their way to the Refuge Area and make contact with the Duty Manager via the intercom system

- The wheel chair user should then wait in the refuge area until either a member of Dorking Halls staff arrive to assist with their evacuation or until members of the emergency services arrive to commence with their evacuation.

4.0 CONTROLLED EVACUATIONS

There are certain situations in which it may be necessary to evacuate the building in a controlled rather than emergency fashion.  Examples are:

- Power cut,  when it has been ascertained that power will be off for a considerable period of time

- Very severe weather warnings

- Other non scheduled closures such as the building being needed as part of a wider ranging emergency plan. Use as a Rest Centre during a major incident, for example.

In the event of any of the above situation all customers are asked to remain where they are aware and await for announcements either by a member of staff or by the Duty Manager via the Tannoy system.

5. Miscellaneous

5.1 Fire Drills

i)  Dorking Halls will organise a fire evacuation drill at least once a year and provide additional refreshment training every six months. If an unscheduled emergency evacuation has occurred that involved a large number of staff then no separately arranged drill may be necessary. Records of all fire drills and unintentional evacuations are kept in the Fire Safety Folder.

ii) Dorking Halls organises a weekly test at a time arranged with the events programme bookings.

iii) The purpose of fire drills is to give members of staff experience in evacuation procedures and to expose any problems with engineering controls or the management of evacuations.

5.2 Fire Prevention Policy and Fire Risk Assessments

i) The Regulatory Reform (Fire Safety) Order 2005 which came into effect on 1 October 2006 requires all employers to carry out fire safety risk assessments of their buildings. Dorking Halls carries out such risk assessments, regularly reviews them and maintains records of them.

5.3 Building Modifications

No modification may be made to the fabric or arrangement of the furnishings in any part of any building without due consideration of fire safety. Proposed changes which may affect a fire escape route may require the permission of local authority Building Control and/or the Fire Brigade which may allow or refuse permission for the change or impose conditions. The Health and Safety Officer liaises with the safety section of the Fire Brigade and can advise on whether proposed changes are likely to require their permission.

5.4 Maintenance of Fire Alarm Systems

All fire alarm systems within the College are subject to regular inspection and maintenance by Dorking Halls Technical staff and contract companies.

5.5 First Aid

i) Dorking Halls maintains a substantial number of employees with 'first aid at work' certificates. It also has many 'emergency responders' (persons with basic emergency first aid training) in low risk areas. An up-to-date list of all first aid personnel is kept in departmental offices. First aiders can be summoned by contacting the Duty Manager by radio or on 3323.

ii) In emergency evacuation situations, all trained first aiders are expected to report to the Duty Manager to enquire whether their assistance is required.

5.6 Management of emergency situations including arrangements for building closures due to loss of critical services, adverse weather, industrial action, etc.

i) Following any emergency situation becoming apparent a meeting the team should refer to the Business Continuity Plan set out in the Duty Manager Folder.

6.1 Bomb Threat.

i) Bomb threats usually take one of two forms. 1) Threats directed against the Local Authority/ Dorking Halls; usually by a telephone caller. 2) Police warnings to the Local Authority/ Dorking Halls; usually about threats to neighbouring properties. In either case the advice of the Police should be sought and/or acted upon. In general, if Dorking Halls has to be wholly or partially evacuated because of a bomb threat the fire alarm is NOT sounded as occupants could unwittingly enter the danger area. Co-opted staff give verbal instructions to occupants starting with any threatened area then working down from the top of buildings. Occupants should be directed away from any declared danger area. As there is no way of knowing exactly which members of staff will be on hand at any particular time, all staff must be prepared to give such instructions to colleagues.

ii) Police advice to the recipient of a bomb threat call is to:

a) keep calm. Try and attract the attention of a colleague while taking the call and indicate the problem by a note or signal.
b) try to obtain as much information as possible especially details of the caller and which building or part of a building is involved.
c) immediately report the incident to the Duty Manager and the police.

6.2 Suspicious packages.

Any member of staff who believes they have detected a suspicious package or a package with suspicious contents should:

a) leave it alone - do not handle to investigate it further.
b) clear and secure the immediate area.
c) dial 3323 or Radio and report the matter to the Duty Manager. The Duty Manager will contact the emergency services and senior Local Authority Officers and act upon their advice. They may also advise the person reporting the incident on further actions to take.
d) in foyer areas, arrangements may have to be made to station staff at access points to direct people to other entrances and exits and routes through the building. As there is no way of knowing exactly which members of staff will be on hand at any particular time, all staff must be prepared to be co-opted to assist in this process.

6.3 Gas Leak.

i) Suspected gas leaks should be reported immediately to the Dorking Halls Technical team via the Radio system or dial 3316. Meanwhile, staff and others who feel they may be at risk are at liberty to leave their area and move to an unaffected part of the building or leave the building completely. Appropriate members of the Dorking Halls Technical team will investigate the reported leak and take appropriate action.

6.4 (Management of emergency situations). This may involve declaring the area safe or the necessity of partially or wholly evacuating the building. Based on this advice the Duty Manager will take the decision whether to evacuate all or part of a building. If this is necessary, co-opted staff will give verbal instructions to occupants; starting at the top of buildings and working down.

ii) In any suspected gas emergency, the Duty Manager should inform the National Grid Gas Emergencies (Formally Transco Gas ) - 0800 111 999 (This is a 24 hour emergency line), arrange for the affected parts or the whole building to be evacuated and call in the relevant maintenance engineer.